LAB EXPERIMENT 12

Aim: To design an ALU (Arithmetic Logic Unit) in XILINX using Verilog language of Coding.

Theory: An arithmetic-logic unit (ALU) is the part of a computer processor (CPU) that carries out arithmetic and logic operations on the operands in computer instruction words. In some processors, the ALU is divided into two units, an arithmetic unit (AU) and a logic unit (LU). The ALU performs simple addition, subtraction, multiplication, division, and logic operations, such as OR and AND. The memory stores the program's instructions and data.

Circuit Diagram:

Truth Table:

Verilog Code of the Program and Outputs

- Verilog Code of the Program:

module ALU_Rahil_062(

input [7:0] a,

input [7:0] b,

input [7:0] c,

output reg [8:0] x,x1,x2,x3,x4,x5,x6,x7

);

always @(a or b)

begin

case(c)

3'd0: x=a+b;

3'd1: x1=a-b;

3'd2: x2=a*b;

3'd3: x3=a>>1; // logical right shifting

3'd4: x4=b<<1; // shifting left

3'd5: x5=a&b;

3'd6: x6=a|b;

3'd7: x7=a^b;

endcase

end

endmodule

- Verilog Code for testbench:

module ALU_testbench_Rahil_062;

// Inputs

reg [7:0] a;

reg [7:0] b;

reg [7:0] c;

// Outputs

wire [8:0] x;

wire [8:0] x1;

wire [8:0] x2;

wire [8:0] x3;

wire [8:0] x4;

wire [8:0] x5;

wire [8:0] x6;

wire [8:0] x7;

// Instantiate the Unit Under Test (UUT)

ALU_Rahil_062 uut (

.a(a),

.b(b),

.c(c),

.x(x),

.x1(x1),

.x2(x2),

.x3(x3),

.x4(x4),

.x5(x5),

.x6(x6),

.x7(x7)

);

initial begin

a=1; b=4; c=0;

#20 c=1; a=12; b=5;

#20 c=2; a=10; b=24;

#20 c=3; a=15; b=36;

#20 c=4; a=20; b=48;

#20 c=5; a=25; b=60;

#20 c=6; a=30; b=72;

#20 c=7; a=35; b=84;

end

endmodule

- Screenshots of the Program and Outputs:

- RTL Schematics: